# RFID Read/Write Devices Development Guide Python

Prepared by Aptar

V0.0.3

RFID Read/Write Devices Development Guide Python	                                             1

1. Preamble	                                                                                                                  7

1.1 General	                                                                                                                  7

1.2 Applicable equipment	7

1.3.	7

1.4. Basic flow of reading and writing	8

2. Quick start	8

3. Description of the connection	9

3.1. RS232 connection	9

3.2. RS485 connection	10

3.3. TCP client connection	10

3.4. TCP Server Listening	10

3.5. USB-HID connection	10

3.6 Closing the connection	12

4. Description of events	12

4.1. ISO18000-6C labeling reporting events	12

4.2. ISO18000-6C labeling end events	13

4.3. ISO18000-6B labeling reporting events	13

4.4. ISO18000-6B Labeling End Events	14

4.5 Incidents of national labeling reporting	14

4.6 End-of-labeling event	15

4.7. GPI triggering of reported events	15

4.8. GPI triggers end-of-report event	16

4.9. TCP Connection Disconnection Events	17

4.10. TCP Connection Events	17

5. Message configuration and query instructions	18

5.1 Sending synchronized messages	18

6. Statement of the message	19

6.1. reader configuration and management	19

6.1.1. restarting the read-write	19

6.1.2. Configuring and Querying Serial Port Parameters	19

6.1.3. Configuring GPO Status Parameters	19

6.1.4. Querying GPI Status Parameters	20

6.1.5. Configuring and Querying GPI Trigger Parameters	20

6.1.6. Querying the software baseband version	21

6.1.7. query reader RFID capability	21

6.1.8. querying reader information	21

6.1.9. Buzzer control	22

6.2. RFID Configuration and Operation	22

6.2.1 Stop command	22

6.2.2. Configuring and querying reader power	23

6.2.3 Configuring and querying the reader's operating band	23

6.2.4. Configuring and Querying EPC Baseband Parameters	23

6.2.5. Configuring and querying GB baseband parameters	24

6.2.6. Configuring and Querying Tag Upload Parameters	24

6.2.7 Reading EPC labels	25

6.2.8 Writing EPC labels	25

6.2.9. lock EPC tags	27

6.2.10 Inactivation of EPC labels	27

6.2.11 Reading 6B labels	28

6.2.12. Write 6B Labeling	28

6.2.13. 6B Tag Locking	29

6.2.14. 6B Tag-Locked Queries	29

6.2.15 Reading GB labels	29

6.2.16. Writing GB tags	30

6.2.17. Lock GB tag	30

6.2.18 Inactivation GB labeling	31

7. Description of parameters	31

7.1.1.  6 C Label Selection Parameters	31

7.1.2.  6 C Tag Reading TID Parameters	33

7.1.3.  6 C Tag Read User Data Area Parameters	33

7.1.4.  6 B Tag Read User Data Area Parameters	33

7.1.5. GB Tag Read User Data Area Parameters	34

8. Appendix 1	34

List of frequency bands supported by the reader	34

# 1. Preamble

## 1.1 General

To facilitate secondary development, we provide a library of functions to run in Python. The library is written in Python and packaged in a standard tar.gz package.

## 1.2 Applicable equipment

| functional module | Applicable equipment type |
| --- | --- |
| Reader Configuration and Management |  |
| RFID Configuration and Operation | All equipment |

### 1.3.

All the contents of the document, including text, pictures are original. The company reserves the right to pursue legal responsibility for unauthorized use in commercial use. Without authorization, the user shall not add, modify, delete the contents of this document, and shall not disseminate it on the Internet, CD-ROM or other means. In case of violation, the consequences will be borne by the user.

## 1.4. Basic flow of reading and writing

# 2. Quick start

from uhf.reader import * def received Epc(epcInfo: LogBaseEpcInfo). if epcInfo.result == 0:. print(epcInfo.epc) def upload EpcOver(epcOver: LogBaseEpcOver):

print("LogBaseEpcOver") if __name__ == '__main__ '. client = GClient() # Serial Connection True is a successful connection if client.openSerial(("COM7", 115200)): # Don't connect repeatedly, call close to free up resources and connect again, the client that is not closed can be reused. # Subscribe to the inventory retracement client.callEpcInfo = received Epc client.callEpcOver = upload EpcOver # Inventory 6c Antenna #1 Cycle Inventory msg = MsgBaseInventoryEpc(EnumG.AntennaNo_1.value, EnumG.InventoryMode_Inventory.value) if client.sendSynMsg(msg) == 0. print(msg.rtMsg) sleep(5) # inventory 5s stop = MsgBaseStop() # stop the inventory if client.sendSynMsg(stop) == 0. print(stop.rtMsg) client.close() # Disconnect to free up resources

# 3. Description of the connection

## 3.1. RS232 connection

| resemble | GClient |
| --- | --- |
| function (math.) | openSerial(readName: (), kwargs) --> bool |
| clarification | readName-->("COM7",115200) Serial port number, baud rate ¦ kwargs-->timeout Default is fine. |

## 3.2. RS485 connection

| resemble | GClient |
| --- | --- |
| function (math.) | openSerial485(readName: (), **kwargs) --> bool |
| clarification | readName-->("COM7",115200,1) serial port number, baud rate, address ¦ kwargs-->timeout Default is fine. |

## 3.3. TCP client connection

| resemble | GClient |
| --- | --- |
| function (math.) | openTcp(readName: (), **kwargs) --> bool |
| clarification | readName-->(" 192.168.1.168 ",8160) ip, port number ¦ kwargs-->timeout silent recognize that |

## 3.4. TCP Server Listening

| resemble | GServer |
| --- | --- |
| methodologies | openServer(port: int) |
| Stop listening | close() |
| clarification | port: The local interface on which the host listens. To listen using this method, you need to configure the UHF read/write device to "Client Mode". The "Client Mode" configuration method is detailed in the RFID Demo Software Operation Manual. |

## 3.5. USB-HID connection

| resemble | GClient |
| --- | --- |
| function (math.) | openUsbHid(param: bytes) --> bool |
| clarification | param-->Can be obtained by getUsbHidPath List() or getUsbHidPathDict(). |

## 3.6 Closing the connection

| function (math.) | client.close() |
| --- | --- |
| clarification | Disconnect Release Resources [This method needs to be called proactively to release resources for linked objects that have expired.] |

# 4. Description of events

## 4.1. ISO18000-6C labeling reporting events

| resemble | GClient |
| --- | --- |
| illustrative example | client.callEpcInfo = receiveEpc def receiveEpc(epcInfo: LogBaseEpcInfo). Pass 6C Tag Master Go Report Event: When the reader is in read state, the tag information will be reported through this event. See "Getting Started" for an example. LogBaseEpcInfo: see "Reporting Objects" for details. |

Reporting Objects

| resemble | LogBaseEpcInfo |
| --- | --- |

| causality | Epc: hexadecimal EPC string BEpc: EPC byte array Pc: PC value AntId: antenna number Rssi: signal strength Result: the result of tag reading, 0 is read successfully, non-0 is failure Tid: hexadecimal TID string BTid: TID byte array Userdata: hexadecimal Userdata string BUser: Userdata byte array Reserved: hexadecimal reserved strings BRes: Byte array of reserved areas |
| --- | --- |
| clarification | 6C Label Active Reporting Parameters. |

## 4.2. ISO18000-6C labeling end events

| resemble | GClient |
| --- | --- |
| illustrative example | client.callEpcOver = receiveEpcOver def receiveEpcOver(epcOver: LogBaseEpcOver). Pass 6C Labels proactively report end parameters to ensure that asynchronous messages get synchronized state |

## 4.3. ISO18000-6B labeling reporting events

| resemble | GClient |
| --- | --- |
| illustrative example | client.call6b Info = receive6b def receive6b(6b Info: LogBase6b Info). pass 6B Tag Master Go Report Event: When the reader is in read state, the tag information will be reported through this event. See "Getting Started" for an example. LogBase6bInfo: See "Reporting objects" for details. |

Reporting Objects

| resemble | LogBase6bInfo |
| --- | --- |
| causality | AntId: antenna number Rssi: signal strength Result: the result of tag reading, 0 is read successfully, non-0 is failure Tid: hexadecimal TID string BTid: TID byte array Userdata: hexadecimal Userdata string BUser: Userdata byte array |
| clarification | 6b Label Active Reporting Parameters. |

## 4.4. ISO18000-6B Labeling End Events

| resemble | GClient |
| --- | --- |
| illustrative example | client.call6bOver = receive6bOver def receive6bOver(6bOver: LogBase6bOver). Pass The 6B tag actively reports the end parameter to ensure that asynchronous messages get a synchronized state. |

## 4.5 Incidents of national labeling reporting

| resemble | GClient |
| --- | --- |
| illustrative example | client.callGb Info = receiveGb Info def receiveGb Info(gb Info: LogBaseGb Info). pass GB Tag master de-report event: When the reader is in read state, the tag information will be reported through this event. See "Getting Started" for an example. LogBaseGbInfo: See "Reporting objects" for details. |

Reporting Objects

| resemble | LogBaseGbInfo |
| --- | --- |
| causality | Epc: hexadecimal EPC string (labeling code area) BEpc: EPC byte array (label encoding area) Pc: PC value AntId: antenna number Rssi: signal strength Result: the result of tag reading, 0 is read successfully, non-0 is failure Tid: hexadecimal TID string (tag information area) BTid: TID byte array (label information area) Userdata: hexadecimal Userdata string BUser: Userdata byte array |
| clarification | Gb Tag Active Reporting Parameters. |

## 4.6 End-of-labeling event

| resemble | GClient |
| --- | --- |
| illustrative example | client.callGbOver = receiveGbOver def receiveGbOver(gbOver: LogBaseGbOver). pass The GB tag proactively reports the end parameter to ensure that asynchronous messages get a synchronized state. |

## 4.7. GPI triggering of reported events

| resemble | GClient |
| --- | --- |

| illustrative example | client.callGpiStart = gpiStart def gpiStart(start: LogGpiStart). pass When the trigger start condition is met, the reader will upload a notification message to inform the host computer that the trigger operation has started LogGpiStart: see "Reporting Objects" for details. |
| --- | --- |

Reporting Objects

| resemble | LogAppGpiStart |
| --- | --- |
| causality | gpiPort: trigger GPI port number gpiPortLevel: GPI port level systemTime: current system time |
| clarification | GPI Trigger Up parameter. |

## 4.8. GPI triggers end-of-report event

| resemble | GClient |
| --- | --- |
| illustrative example | client.callGpiOver = gpiOver def gpiOver(over: LogGpiOver). pass When the trigger stop condition is met, the reader will upload a notification message to inform the host computer that the trigger operation has started LogGpiOver: see "Reporting Objects". |

Reporting Objects

| resemble | LogAppGpiOver |
| --- | --- |

| causality | gpiPort: trigger GPI port number gpiPortLevel: GPI port level systemTime: current system time |
| --- | --- |
| clarification | GPI Trigger Stop Reporting parameter. |

## 4.9. TCP Connection Disconnection Events

| resemble | GClient |
| --- | --- |
| typical example | client.callTcpDisconnect = tcpDisconnect def tcpDisconnect(readerName: ()). pass |
| clarification | Description: > The connection is in TCP and this event is reported when the remote connection is actively disconnected or when there is a physical layer exception. > After this event is reported, the host computer (caller) must release the connection object, otherwise the event will be reported in a loop until the connection object is released. > Whether to reconnect to this remote device is under the control of the host computer (caller) to fulfill different requirements. readerName: name of the connection object, ('192.168.1.168', 8160). |

## 4.10. TCP Connection Events

| resemble | GServer |
| --- | --- |
| typical example | g_server.callGClientConnected = clientConnected def clientConnected(readerSerialNumber: str, gClient: GClient). pass |
| clarification | TCP is in the listening state, when the remote read/write device actively connects to the host computer, this event will be triggered to report. readerSerialNumber:Unique Serial Number gClient: See "Who to report" for details. |

Reporting Objects

| resemble | GClient |
| --- | --- |

| clarification | Note: This connection object is identical to the other actively connected objects and is used in exactly the same way. |
| --- | --- |

# 5. Message configuration and query instructions

## 5.1 Sending synchronized messages

| resemble | GClient |
| --- | --- |
| methodologies | def sendSynMsg(msg: Message, timeout=3) Synchronize message Wait for return value |
| Method 1 | def senUnSynMsg(msg: Message) Unsynchronized message Do not wait for return value. |
| return value | msg.rtCode: message return code 0 for operation success, non-0 operation failure. msg.rtMsg: return operation information in case of communication success, return None in case of communication failure. |
| clarification | Messages such as "Reader Configuration and Management" and "RFID Configuration and Operation" are sent by this method. |

Code Example 1

// Stop command, idle state stop = MsgBaseStop() if client.sendSynMsg(stop) == 0: # shortcut judgment writing print(stop.rtMsg)

Code Example 2

// For power configuration, set all 4 antennas to 30dBm. dicPower = {"1": 30, "2": 30, "3": 30, "4": 30} msg = MsgBaseSetPower(**dicPower) if client.sendSynMsg(msg) == 0. print(msg.rtMsg)

# 6. Statement of the message

## 6.1. reader configuration and management

### 6.1.1. restarting the read-write

| resemble | MsgAppReset |
| --- | --- |
| causality | not have |
| clarification | A device reboot message, typically executed after modifying a configuration that requires a reboot to take effect. |

### 6.1.2. Configuring and Querying Serial Port Parameters

| configuration class | MsgAppSetSerialParam(baudRateIndex: int) |
| --- | --- |
| query class | MsgAppGetSerialParam |
| causality | baudRateIndex: baud rate index (0, 9600 bps; 1, 19200 bps; 2, 115200 bps; 3, 230400) bps; 4 , 460800bps) |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Configure the device serial port parameters. |

## 6.1.3. Configuring GPO Status Parameters

| configuration class | MsgAppSetGpo(**kwargs) |
| --- | --- |
| causality | gpo1: 0 (low, relay open) 1 (high, relay closed) gpo2: 0 (low, relay open) 1 (high, relay closed) gpo3: 0 (low, relay open) 1 (high, relay closed) gpo4: 0 (low, relay open) 1 (high, relay closed) |

| clarification | (Persistent Configuration, Power-Off Save) Configures the device GPO parameters. Note: For GPOs that do not require a control state, no assignment is necessary. |
| --- | --- |

## 6.1.4. Querying GPI Status Parameters

| query class | MsgAppGetGpiState |
| --- | --- |
| causality | dicGpi: level state of the corresponding GPI ( type: dict , key: GPI index number, value: level state (0 low, 1 high) ) |
| clarification | Queries the device GPI status. Note: Index numbers begin with 1. |

### 6.1.5. Configuring and Querying GPI Trigger Parameters

| configuration class | MsgAppSetGpiTrigger |
| --- | --- |
| query class | MsgAppGetGpiTrigger |
| causality | gpiPort: GPI port number, indexed from 0 triggerStart: trigger start (0 trigger off, 1 low trigger, 2 high trigger, 3 rising edge trigger, 4 falling edge trigger, 5 any edge trigger) hexTriggerCommand: trigger binding command (Hex, can be empty) triggerOver: trigger stop (0 no stop, 1 low trigger, 2 high trigger, 3 rising edge trigger, 4 falling edge trigger, 5 any edge trigger, 6 delay stop) overDelayTime: delayed stop time (in effect only if the stop condition is "delayed stop") levelUploadSwitch:Trigger the IO level change upload switch when not stopped (0 no upload, 1 upload) |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Configure the device GPI trigger parameters. Note: This configuration needs to be modified when the device is idle (i.e., the cyclic card reading state cannot change the configuration). |

### 6.1.6. Querying the software baseband version

| query class | MsgAppGetBaseVersion |
| --- | --- |
| causality | baseVersions: baseband software versions |
| clarification | Used to obtain the version number of the baseband software. |

### 6.1.7. query reader RFID capability

| query class | MsgBaseGetCapabilities |
| --- | --- |
| causality | MaxPower: Maximum supported power MinPower: Minimum supported power AntennaCount: number of antennas FrequencyArray: list of supported frequency bands. 0, National Standard 920~925MHz 1, National Standard 840~845MHz 2 ， 国标 840~845MHz 和 920~925MHz 3, FCC, 902~928MHz 4, ETSI, 866~868MHz ProtocolArray: list of supported protocols. 0, ISO18000-6C/ EPC C1G2 1, ISO 18000-6B 2, National Standard GB/T 29768-2013 3, National Military Standard GJB 7383.1-2011 |
| clarification | None. |

### 6.1.8. querying reader information

| query class | MsgAppGetReaderInfo |
| --- | --- |

| causality | readerSerialNumber: reader-writer serial number powerOnTime: power-on time baseCompileTime: baseband compile time appVersions: application software versions (e.g., "0.1.0.0") appCompileTime: application compile time systemVersions: operating system versions |
| --- | --- |
| clarification | None. |

## 6.1.9. Buzzer control

| query class | MsgAppSetBeep |
| --- | --- |
| causality | beepStatus: 0- Stop 1- Loud beepMode: 0- beep once 1- beep often |
| clarification | None. |

### 6.2. RFID Configuration and Operation

## 6.2.1 Stop command

| resemble | MsgBaseStop |
| --- | --- |
| causality | not have |
| clarification | Stops all RFID operations of the reader and puts the reader into an idle state. Tip: When the reader is in the read state, all configuration messages will not be sent and a stop command must be sent. |

### 6.2.2. Configuring and querying reader power

| configuration class | MsgBaseSetPower |
| --- | --- |
| query class | MsgBaseGetPower |
| causality | dicPower : the corresponding antenna power of the reader ( type: dict, key: antenna index number, value: antenna power value ) |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Used to configure the current power of the reader/writer. |
| Configuration example | dicPower = {"1": 30, "2": 30, "3": 30, "4": 30} msg = MsgBaseSetPower(**dicPower) if client.sendSynMsg(msg) == 0. print(msg.rtMsg) |

### 6.2.3 Configuring and querying the reader's operating band

| configuration class | MsgBaseSetFreqRange |
| --- | --- |
| query class | MsgBaseGetFreqRange |
| causality | freqRangeIndex: band index, see Appendix 1 for details. |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Used to configure the current operating band of the reader/writer. |

### 6.2.4. Configuring and Querying EPC Baseband Parameters

| configuration class | MsgBaseSetBaseband |
| --- | --- |
| query class | MsgBaseGetBaseband |

| causality | baseSpeed: EPC baseband rate (optional). qValue: Default Q value (optional) (0~15). session: (optional)(0,Session0; 1,Session1; 2,Session2; 3,Session3). inventoryFlag: inventory flag parameter (optional) (0, only use Flag A for inventory; 1, only use Flag B for inventory; 2, use Flag A and Flag B in turn). |
| --- | --- |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Used to configure the baseband parameters used by the reader. |

### 6.2.5. Configuring and querying GB baseband parameters

| configuration class | MsgBaseSetGbBaseband |
| --- | --- |
| query class | MsgBaseGetGbBaseband |
| causality | speed_tc: 0, Tc=6.25us; 1, Tc=12.5us speed_trext:0 , Trext=0, no leading tone; 1 , Trext=1, leading tone speed_k:reverse link factor K , 0, K=1/5; 1, K=3/7; 2, K=6/ 11; 3, K= 1; 4, K=2/5; 5. K=6/7; 6, K= 12/ 11; 7, K=2; 8~15, retained speed_miller:reverse link code, 0 , FM0; 1 , Miller=2; 2 , Miller=4; 3 , Miller=8 cin:Anti-collision parameter, Continuous Idle Threshold, CIN , default value is 4 ccn:anti-collision parameter, continuous collision threshold, CCN , default value is 3 session: (0,Session0; 1,Session1; 2,Session2; 3,Session3). (Optional) inventoryFlag: inventory flag parameter (0, only use Flag A inventory; 1, only use Flag B inventory; 2, use Flag A and Flag B in turn). (Optional) |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Used to configure the GB baseband parameters used by the reader. |

### 6.2.6. Configuring and Querying Tag Upload Parameters

| configuration class | MsgBaseSetTagLog |
| --- | --- |
| query class | MsgBaseGetTagLog |

| causality | RepeatedTime: Repeated tag filtering time (optional) (Indicates that the same tag content will be uploaded only once within the specified repeated filtering time within one read command execution cycle, 0~65535, time unit: 10ms). RssiTV: RSSI Threshold (optional) (tag data will not be uploaded and discarded if the tag RSSI value falls below the threshold). |
| --- | --- |
| clarification | Persistent configuration, power off to save, please do not repeat the configuration operation, need to be frequently configured, please query the check first! Used to configure the read-write upload parameters. |

## 6.2.7 Reading EPC labels

| resemble | MsgBaseInventoryEpc |
| --- | --- |
| causality | antennaEnable: antenna port (use antenna constants, see Quick Start) inventoryMode: Continuous/Single Read (0: Single Read mode, the reader performs one round of reading on each enabled antenna and then terminates the reading operation and automatically enters the idle state; 1: Continuous Read mode, the reader performs the reading operation until the reader receives a stop command and terminates the reading) filter: Selection of reading parameters (optional) (see parameter description) readTid: TID read parameter (optional) (see parameter description for details) readUserdata: User data area read parameter (optional) (see parameter description for details) readReserved: Parameter for reserved area read (optional) (see parameter description for details) hexPassword: access password (optional) |
| clarification | Used to configure the tag reading parameters of the reader and start the card reading operation, any operation to read the tag data needs to get the tag EPC code first, so any card reading operation will get the EPC code. |

## 6.2.8 Writing EPC labels

| resemble | MsgBaseWriteEpc |
| --- | --- |
| causality | AntennaEnable: Antenna Port Area: label data area to be written (0, reserved area; 1, EPC area; 2, TID area; 3, user data area) Start: start address of the word to be written to the label data area HexWriteData: data content to be written (optional) (hexadecimal) BwriteData: content of data to be written Filter: Selection of reading parameters (optional) (see parameter description) HexPassword: access password (optional) |

| clarification | > The reader performs a write operation on the EPC label, and the write operation defined by this instruction is a single operation. > The ISO18000-6C protocol specifies a minimum data unit of words for read and write operations. > EPC zone consists of CRC- 16(0th word) + PC(1st word) + EPC: CRC16: 0th character, not writable. PC: the first word, the first 5 bit bits represent the EPC word length, i.e., PC is calculated by shifting the word length of EPC to the left by 11 bits. |
| --- | --- |
| Write results | 0, Write success 1, Antenna port parameter error 2, Wrong choice of parameters 3, Write parameter error 4, CRC Checksum Error 5, Insufficient power 6, Data area overflow 7, data area locked 8, Access password error 9, Other labeling errors 10, label missing 11 , Read-write send command error |

### 6.2.9. lock EPC tags

| resemble | MsgBaseLockEpc |
| --- | --- |
| causality | AntennaEnable: Antenna Port Area: Tag data area to be locked (0, inactivation password area; 1, access password area; 2, EPC area; 3, TID area; 4, user data area) Mode: lock operation type (0, unlocked; 1, locked; 2, permanently unlocked; 3, permanently locked) Filter: Selection of reading parameters (optional) (see parameter description) HexPassword: access password (optional) |
| clarification | Perform lock or unlock operation on the tag, the operation defined by this instruction is a single operation |

## 6.2.10 Inactivation of EPC labels

| resemble | MsgBaseDestoryEpc |
| --- | --- |

| causality | AntennaEnable: Antenna Port HexPassword: Destroy password Filter: Selection of reading parameters (optional) (see parameter description) |
| --- | --- |
| clarification | Inactivate the label, the label will be permanently deactivated after inactivation, this operation is irreversible. The operation defined in this instruction is a single operation |

## 6.2.11 Reading 6B labels

| resemble | MsgBaseInventory6b |
| --- | --- |
| causality | AntennaEnable: Antenna Port InventoryMode: Continuous/Single Read (0: Single Read mode, the reader will do one round of reading on each enabled antenna and then end the reading operation and enter the idle state automatically; 1: Continuous Read mode, the reader will keep reading until it receives a stop command to end the reading) Area: read content (0, read 6B TID only; 1, read 6B TID + user data; 2, read user data only) ReadUserdata: User data area read parameter (optional) (see parameter description for details) HexMatchTid: TID code of the 6B tag to be matched (optional) (hexadecimal) BMatchTid: TID code of the 6B tag to be matched (optional) |
| clarification | Data read operation for ISO18000-6B labels |

## 6.2.12. Write 6B Labeling

| resemble | MsgBaseWrite6b |
| --- | --- |
| causality | AntennaEnable: Antenna Port HexMatchTid: TID code of the 6B tag to be matched (optional) (hexadecimal) BMatchTid: TID code of the 6B tag to be matched. Start: start address of the byte to be written to the label data area HexWriteData: data content to be written (optional) (hexadecimal) BwriteData: content of data to be written |
| clarification | Write operation to the 6B tag, the write operation defined by this instruction is a single operation |

### 6.2.13. 6B Tag Locking

| resemble | MsgBaseLock6b |
| --- | --- |
| causality | AntennaEnable: Antenna Port HexMatchTid: TID code of the 6B tag to be matched (optional) (hexadecimal) BMatchTid: TID code of the 6B tag to be matched. LockIndex: byte address of the data to be locked |
| clarification | The lock operation on 6B tag data is irrevocable and irreversible, and the lock operation defined in this instruction is a single operation. |

### 6.2.14. 6B Tag-Locked Queries

| resemble | MsgBaseLock6bGet |
| --- | --- |
| causality | AntennaEnable: Antenna Port HexMatchTid: TID code of the 6B tag to be matched (optional) (hexadecimal) BMatchTid: TID code of the 6B tag to be matched. LockIndex: byte address of the query data to be locked |
| clarification | To query the lock status of 6B tag data, the lock query operation defined in this instruction is a single operation. |

## 6.2.15 Reading GB labels

| resemble | MsgBaseInventoryGb |
| --- | --- |

| causality | AntennaEnable: Antenna port (uses the antenna enumeration, see Quick Start for details) InventoryMode: Continuous/Single Read (0: Single Read mode, the reader will do one round of reading on each enabled antenna and then end the reading operation and enter the idle state automatically; 1: Continuous Read mode, the reader will keep reading until it receives a stop command to end the reading) Filter: Selection of reading parameters (optional) (see parameter description) ReadTid: TID read parameter (optional) (see parameter description for details) ReadUserdata: User data area read parameter (optional) (see parameter description for details) HexPassword: access password (optional) |
| --- | --- |
| clarification | It is used to configure the tag reading parameters of the reader and start the reading operation. Any operation to read the tag data needs to get the tag code first, so any reading operation will get the tag code (EPC). |

## 6.2.16. Writing GB tags

| resemble | MsgBaseWriteGb |
| --- | --- |
| causality | AntennaEnable: Antenna port (uses the antenna enumeration, see Quick Start for details) Area: Label data area to be written (0x10, label code area 0x20, label security area 0x30~0x3F, user sub-area 0~15) Start: start address of the word to be written to the label data area HexWriteData: data content to be written (optional) (hexadecimal) BwriteData: content of data to be written Filter: Selection of reading parameters (optional) (see parameter description) HexPassword: access password (optional) |
| clarification | > The reader performs a write operation on the GB tag, and the write operation defined by this instruction is a single operation. > The GB protocol specifies that the minimum data unit for read and write operations is a word. |

## 6.2.17. Lock GB tag

| resemble | MsgBaseLockGb |
| --- | --- |

| causality | AntennaEnable: Antenna port (uses the antenna enumeration, see Quick Start for details) Area: Tag data area to be locked (0x00 Tag Information Area, 0x10 Tag Encoding Area, 0x20 Tag Security Area, 0x30~0x3F User Sub Area 0~15) Mode: Lock operation type 0x00 , readable and writable. 0x01 , readable and non-writable. 0x02 , not readable or writable. 0x03 , not readable or writable. 0x11 , the security mode is set to No Authentication Required; this operation area must be a label security area. 0x12 , the security mode is set to require authentication, no secure communication; this operation area must be the label security zone. 0x13 , the security mode is set to Requires authentication, secure communication is required; this operation area must be the label security area. Filter: Selection of reading parameters (optional) (see parameter description) HexPassword: access password (optional) |
| --- | --- |
| clarification | Perform lock or unlock operation on the tag, the operation defined by this instruction is a single operation |

## 6.2.18 Inactivation GB labeling

| resemble | MsgBaseDestoryGb |
| --- | --- |
| causality | AntennaEnable: Antenna port (uses the antenna enumeration, see Quick Start for details) HexPassword: Destroy password Filter: Selection of reading parameters (optional) (see parameter description) |
| clarification | Inactivate the label, the label will be permanently deactivated after inactivation, this operation is irreversible. The operation defined in this instruction is a single operation |

# 7. Description of parameters

## 7.1.1.  6 C Label Selection Parameters

| resemble | ParamEpcFilter |
| --- | --- |

| causality | Area: the data area to be matched (1, EPC area; 2, TID area; 3, user data area) BitStart: Match data start bit address BitLength: length of the data bits to be matched HexData: data content to be matched (optional) ( hexadecimal ) |
| --- | --- |
| clarification | Optional parameter (the first 32 bits of the EPC are the PC value, so the start bit address when distinguishing the EPC is usually 32) |

## 7.1.2.  6 C Tag Reading TID Parameters

| resemble | ParamEpcReadTid |
| --- | --- |
| causality | Mode: TID reading mode configuration, (0, TID reading length is adaptive, but the maximum length does not exceed the length defined by byte 1; 1, read TID according to the length defined by byte 1) Len: the length of the word (word, 16bits, the same below) that the reader needs to read the TID data. |
| clarification | Optional parameters |

## 7.1.3.  6 C Tag Read User Data Area Parameters

| resemble | ParamEpcReadUserdata |
| --- | --- |
| causality | Start: Start word address Len: the word length of the user data to be read by the reader. |
| clarification | Optional parameters |

## 7.1.4.  6 B Tag Read User Data Area Parameters

| resemble | Param6bReadUserdata |
| --- | --- |
| causality | Start: User data start byte address Len: length of user data byte |
| clarification | Optional parameters |

## 7.1.5. GB Tag Read User Data Area Parameters

| resemble | ParamGbReadUserdata |
| --- | --- |
| causality | ChildArea: user child area Start: user data start word address Len: user data word length |
| clarification | Optional parameters |

# 8. Appendix 1

# List of frequency bands supported by the reader

| indexing | clarification |
| --- | --- |
| 0 | National Standard 920~925MHz |
| 1 | National Standard 840~845MHz |
| 2 | National Standard 840~845MHz and 920~925MHz |
| 3 | FCC, 902~928MHz |
| 4 | ETSI, 866~868MHz |